Lab

Sift & Harris Corners

Part 1: SIFT

- Using SIFT and showing keys:

  - Apply sift function on example.

  - What does the result of the function mean?

  - Apply show keys function on the result from sift

  - What does the result image mean?

- Object Recognition using SIFT:

  - Try match function between (‘box_in_scene.png’, ‘box.png’)

- Explain matching process main steps and how can I use other feature extractors.

Part 2: Harris Corner Detection

  - Read the image ‘circuit.tif’

  - Compute x and y Derivatives of image

Ix=Gσx*I,  Iy=Gσy*I

  - Compute the products of derivatives at every pixel

Ix2=Ix.Ix,  Ixy=Ix.Iy,  Iy2=Iy.Iy

  - Compute the sum of the products of derivatives at each pixel using Gaussian filter

Sx2=Gσ*Ix2,  Sxy=Gσ*Ixy,  Sy2=Gσ*Iy2

  - Compute the response of the detector at each pixel

R=(Sx2.Sy2-Sxy.Sxy)/ (Sx2+ Sy2)

  - Apply Threshold on R to get corners (R > T is a corner).

  - Get local maximum of R (after applying the threshold) to remove redundant points (only leave points that is larger than all surrounding points).

  - According the TODOs.

Hints:

Sobel Operator: Gx=-101-202-101  Gy=-1-2-1000121

References:

https://docs.opencv.org/3.0-beta/doc/py_tutorials/py_feature2d/py_matcher/py_matcher.html